Safety Talks – Activity Package 3

Mobile Equipment Access and Egress

Mounting and dismounting equipment is one of the most dangerous activities associated with equipment.

The reason for this is due to a variety of factors:

- Rushing and not paying attention to foot or hand placement.

- Slippery or obstructed surfaces.

Things You Can Do to Reduce Injuries Due to Mounting and Dismounting:

- Always read the Operation & Maintenance Manual for proper mount and dismount procedures.

- Always Inspect the ground before climbing up or down. Note where feet will be placed to avoid a twisted or sprained ankle.

- Always check the condition of the steps, ladders and rails for mud, water, ice, dust or any other material that could cause slips.

- Always maintain three points of contact when mounting and dismounting equipment – This means always have two feet and one hand or two hands and one foot in contact with the machine at all times

  - Doing so means not carrying items when mounting or dismounting equipment.

  - If carrying items up and down from the cab is necessary, place the item(s) on a bench or a ledge of the equipment and stagger step the items up or down with the extra hand while maintaining three points of contact, or you can also use a rope to raise or lower the items.

- Always mount and dismount while facing the equipment.

- Inspect the condition of rails and guarding for damage and effectiveness.

- Always close and latch gates, as required, for fall protection.

- Close doors to the equipment in case the outside grab bars are used as handles while climbing in or out

- DO NOT RUSH – Take the time needed to properly enter and exit the equipment; do not skip steps or rungs in the ladder.

Questions to Generate Discussion:

- What is the top reason for slips and trips while mounting and dismounting your equipment?

- In what condition are the steps, ladders, and handrails on the machines that you use?

*Special thanks to SAFETY.CAT.COM for the above information

Safety Talks

| MEETING DESCRIPTION |  |  |  |
| --- | --- | --- | --- |
| Meeting Date: |  | Time: |  |
| Location: |  | Supervisor: |  |
| Number in Crew: |  | Number Attended: |  |
| Attended By: |  |  | Absent |

| REVIEW ITEMS FROM PREVIOUS MEETING | INCIDENTS/INJURIES REVIEWED |
| --- | --- |

TOPICS DISCUSSED

- Access and Egress from Equipment

WORKERS CONCERNS

CORRECTIVE ACTIONS TO BE TAKEN

CORRECTIVE ACTIONS TO BE TAKEN

| CORRECTIVE ACTIONS TO BE TAKEN |  |  |  |
| --- | --- | --- | --- |
| Supervisor: |  | Manager: |  |
| Date: |  | Date: |  |