Ladder Safety

| Corporate: |  | Practice |
| --- | --- | --- |
| Business Unit and/ or Function | ALL | Number: |
| Subject: | LADDER SAFETY | Reference: |

| 1. Purpose | Ladders are utilized on a daily basis for access and egress to work areas and to a more limited sense, to work from. |
| --- | --- |
| 2. Definitions | None |
| 3. Hazards | - Falls - Falling objects - Contact with energized equipment - Access/egress - Structural failure of ladder |
| 4. Prerequisites | Conduct a risk assessment prior to utilization |
| 5. Precautions | - Ensure ladders are in good condition, damaged ladders are to be removed from service and immediately repaired or replaced. - For every 4 units up ensure that the ladder base is 1 out from the wall. - Make sure there is enough room to safely step off. - Metal ladder conduct electricity, keep them away from power lines and electricity. - Make sure when using a extension ladder that two sections of the ladder overlap according to the manufacturer’s instruction. - Check to make sure that extension ladder locks are properly engaged. - Step ladder must never be used folded up and leaning against a surface. - Be very cautious getting on and off equipment - When going up or down ladders; always face the ladder; keep both hands free and maintain three point contact at all times. |
|  | - Do not work from the top two rungs of any ladder. - The top cap is not a step. - Only one worker shall be on a ladder at any time. - Look up/down before accessing. - Set up barricades or warnings around ladder if it must be set close to thoroughfares or passageway doors. - Ladders shall be set up on a firm, level surface. - Keep the centre of your body within the side rails of the ladder, do not over-reach or extend. - Do not use makeshift items such as barrels or pails as a substitute for a ladder. - Extend ladders at least one meter (3 rungs) beyond the dismount point. - When extended, the overlap of upper and lower sections must not be less than one meter. - Set the base of the ladder with one foot horizontal for every four feet vertical. - Secure the ladder at the top and bottom by tying it off or have another worker hold the ladder. - Step ladders must only be used in the fully opened position with the spreader bars locked in place. - Stepladders are not to be used as support for scaffold planks. - Clean boots prior to using ladders. - Use CSA/ANS, approved spike foot ladders for soft surfaces; non-slip foot ladders for hard, smooth surfaces and non-conductive ladders for electrical work. |
| 6. Interpretation & Updating | To be implement (if required) within the Business Units and/or Functions. This is a draft example only. |